# Conexión desde Python a MQTT

https://www.hivemq.com/article/mqtt-client-library-paho-python/

1.- Descarga e instala Python, desde Python.org

2.- Descarga e instala VSCode desde https://code.visualstudio.com/

3.- Registra una cuenta en hivemq.com

Datos de conexión

4.- Genera un programa en Python, que publique un dato en el servidor hivemq.com, por medio de “paho mqtt client”, para Python.

Desde un terminal, instalar “pip install paho-mqtt” o bien, desde la ruta en donde se encuenta Python instalado, “Python -m pip install paho-mqtt”

Obtenido desde: https://github.com/hivemq-cloud/paho-mqtt-client-example/tree/master

import time

import paho.mqtt.client as paho

from paho import mqtt

# setting callbacks for different events to see if it works, print the message etc.

def on_connect(client, userdata, flags, rc, properties=None):

"""

Prints the result of the connection with a reasoncode to stdout ( used as callback for connect )

:param client: the client itself

:param userdata: userdata is set when initiating the client, here it is userdata=None

:param flags: these are response flags sent by the broker

:param rc: stands for reasonCode, which is a code for the connection result

:param properties: can be used in MQTTv5, but is optional

"""

print("CONNACK received with code %s." % rc)

print("Conectando a MQTT")

# with this callback you can see if your publish was successful

def on_publish(client, userdata, mid, properties=None):

"""

Prints mid to stdout to reassure a successful publish ( used as callback for publish )

:param client: the client itself

:param userdata: userdata is set when initiating the client, here it is userdata=None

:param mid: variable returned from the corresponding publish() call, to allow outgoing messages to be tracked

:param properties: can be used in MQTTv5, but is optional

"""

print("mid: " + str(mid))

# print which topic was subscribed to

def on_subscribe(client, userdata, mid, granted_qos, properties=None):

"""

Prints a reassurance for successfully subscribing

:param client: the client itself

:param userdata: userdata is set when initiating the client, here it is userdata=None

:param mid: variable returned from the corresponding publish() call, to allow outgoing messages to be tracked

:param granted_qos: this is the qos that you declare when subscribing, use the same one for publishing

:param properties: can be used in MQTTv5, but is optional

"""

print("Subscribed: " + str(mid) + " " + str(granted_qos))

# print message, useful for checking if it was successful

def on_message(client, userdata, msg):

"""

Prints a mqtt message to stdout ( used as callback for subscribe )

:param client: the client itself

:param userdata: userdata is set when initiating the client, here it is userdata=None

:param msg: the message with topic and payload

"""

print(msg.topic + " " + str(msg.qos) + " " + str(msg.payload))

# using MQTT version 5 here, for 3.1.1: MQTTv311, 3.1: MQTTv31

# userdata is user defined data of any type, updated by user_data_set()

# client_id is the given name of the client

client = paho.Client(client_id="19b934c3aad04cc7898947a01eb5b752.s1.eu.hivemq.cloud", userdata=None, protocol=paho.MQTTv5)

client.on_connect = on_connect

# enable TLS for secure connection

client.tls_set(tls_version=mqtt.client.ssl.PROTOCOL_TLS)

# set username and password

client.username_pw_set("miles_test", ".Test.2023.")

# connect to HiveMQ Cloud on port 8883 (default for MQTT)

client.connect("19b934c3aad04cc7898947a01eb5b752.s1.eu.hivemq.cloud", 8883)

# setting callbacks, use separate functions like above for better visibility

client.on_subscribe = on_subscribe

client.on_message = on_message

client.on_publish = on_publish

# subscribe to all topics of encyclopedia by using the wildcard "#"

client.subscribe("sensores/#", qos=1)

# a single publish, this can also be done in loops, etc.

client.publish("sensores/temperatura", payload="cold", qos=1)

# loop_forever for simplicity, here you need to stop the loop manually

# you can also use loop_start and loop_stop

client.loop_forever()